PRODUCT SPECIFICATION

PRODUCT SPECIFICATION

Preliminary

- P/N：PO-MT3535CWSN-A01

| CUSTOMER APPROVED SIGNATURES | APPROVED BY | CHECKED BY | PREPARED BY |
| --- | --- | --- | --- |

- Features

- Small package with high efficiency

RoHs Compliant and Pb-Free

- Energy Star / ANSI compliant binning structure

- Application

- Tiny illumination

- PAR 30 light bulb

- Spot light

- Indicator

- Absolute Maximum Ratings

（Ta=25℃）

| Parameter | Symbol | Rating | Unit |
| --- | --- | --- | --- |
| Max Pulse Power Dissipation | PD | 3 | W |
| Forward Current (DC) | IF | 700 | mA |
| Forward Voltage (DC) | VF | 3.6 | V |
| Reverse Current | IR | 85 | mA |
| Operating Temperature | Topr | -40 ~ +100 | ℃ |
| Storage Temperature | Tstg | -40 ~ +100 | ℃ |
| LED junction temperature | Tj | 120 | ℃ |

- Mechanical Dimension

Cathode Mark

Cathode Mark

(unit:mm)

Note:

- Dimensions are in millimeters.

- Tolerance +/- 0.2 mm.

- Luminous Flux Characteristics

（Ta=25℃）

| Color | CCT | Part No. | CRI | Luminous Flux (lm) |  |  | Condition |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Ra | Min | Type | Max |  |
| Cool White | 5700K | PO-MT3535CWSN-A01 | 70 | 90 | / | 140 | IF=350mA |

Note:

- Maintains a tolerance of ±10% on Luminous Flux measurements.

- Maintains a tolerance of ±2% on CRI measurements.

- Optical Characteristics

（Ta=25℃）

| Color | Part No. | Color Temperature |  |  | Typical View Angle (Degrees) 2θ ½ |
| --- | --- | --- | --- | --- | --- |
|  |  | Min. | Typ. | Max. |  |
| Cool White | PO-MT3535CWSN-A01 | 4745 | / | 7040 | 120∘ |

Note:

- Maintains a tolerances of ±5% for CCT measurements.

- Electrical Characteristics

Ta=25℃）

| Color | Part No. | Forward Voltage VF(V) |  |  | Condition |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Type | Max |  |
| Cool White | PO-MT3535CWSN-A01 | 2.9 | / | 3.6 | IF=350mA |

Note:

1.  Maintains a tolerance of ±1v on Forward Voltage measurements.

- Bin Range Of Luminous Intensity

| Bin | Min | Max | Unit | Condition |
| --- | --- | --- | --- | --- |
| AB | 90 | 100 | lm | IF=350mA |
| AC | 100 | 110 |  |  |
| AD | 110 | 120 |  |  |
| AE | 120 | 130 |  |  |
| AF | 130 | 140 |  |  |

- Bin Range Of Forward Voltage

| Bin | Min | Max | Unit | Condition |
| --- | --- | --- | --- | --- |
| H1 | 2.9 | 3.0 | V | IF=350mA |
| H2 | 3.0 | 3.1 |  |  |
| I1 | 3.1 | 3.2 |  |  |
| I2 | 3.2 | 3.3 |  |  |
| J1 | 3.3 | 3.4 |  |  |
| J2 | 3.4 | 3.5 |  |  |
| K2 | 3.5 | 3.6 |  |  |

- Typical Electro-Optical-Thermal Characteristics Curves

(TA=25℃ Unless Otherwise Noted)

| 1.Relative Luminous Flux vs. Forward Current | 2.Forward Voltage vs. Forward Current |
| --- | --- |
| 3.Relative Luminous Flux vs. Ambient Temperature | 4.Forward Voltage vs. Ambient Temperature |
| 5.Spectrum Distribution-CRI 70 | 6. Radiation Diagram |

- ANSI Chromaticity Bin Table

Cool White BIN Structure

Note :The red dashed circle is main bin

| Bin code | CIE-x | CIE-y |  | Bin code | CIE-x | CIE-y |  | Bin code | CIE-x | CIE-y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65A | 0.3068 | 0.3113 |  | 57A | 0.3222 | 0.3243 |  | 50A | 0.3371 | 0.3493 |  |
|  | 0.3145 | 0.3187 |  |  | 0.3294 | 0.3306 |  |  | 0.3452 | 0.3558 |  |
|  | 0.3131 | 0.3290 |  |  | 0.3293 | 0.3423 |  |  | 0.3441 | 0.3428 |  |
|  | 0.3048 | 0.3209 |  |  | 0.3215 | 0.3353 |  |  | 0.3366 | 0.3369 |  |
| 65B | 0.3048 | 0.3209 |  | 57B | 0.3215 | 0.3353 |  | 50B | 0.3376 | 0.3616 |  |
|  | 0.3131 | 0.3290 |  |  | 0.3293 | 0.3423 |  |  | 0.3464 | 0.3688 |  |
|  | 0.3117 | 0.3393 |  |  | 0.3292 | 0.3539 |  |  | 0.3452 | 0.3558 |  |
|  | 0.3028 | 0.3304 |  |  | 0.3207 | 0.3462 |  |  | 0.3371 | 0.3493 |  |
| 65C | 0.3131 | 0.3290 |  | 57C | 0.3292 | 0.3539 |  | 50C | 0.3464 | 0.3688 |  |
|  | 0.3213 | 0.3371 |  |  | 0.3293 | 0.3423 |  |  | 0.3551 | 0.3760 |  |
|  | 0.3205 | 0.3481 |  |  | 0.3371 | 0.3493 |  |  | 0.3533 | 0.3624 |  |
|  | 0.3117 | 0.3393 |  |  | 0.3376 | 0.3616 |  |  | 0.3452 | 0.3558 |  |
| 65D | 0.3145 | 0.3187 |  | 57D | 0.3294 | 0.3306 |  | 50D | 0.3452 | 0.3558 |  |
|  | 0.3221 | 0.3261 |  |  | 0.3366 | 0.3369 |  |  | 0.3533 | 0.3624 |  |
|  | 0.3213 | 0.3371 |  |  | 0.3371 | 0.3493 |  |  | 0.3515 | 0.3487 |  |
|  | 0.3131 | 0.3290 |  |  | 0.3293 | 0.3423 |  |  | 0.3441 | 0.3428 |  |

Note:

Maintains a tolerances of ±0.01 on Color coordinates measurements.

- PACKAGING

PO-MT3535CWSN-A01 High Power LEDs are packaged in reel of 1000 paces and placed inside a carton.

LABEL

BIN CODE

Lot

Qty

LABEL

BIN CODE

Lot

Qty

Carton

CARTON